# Analysis of DIA/SWATH data in Skyline

Note: This tutorial uses data from a Sciex TripleTOF 6600 instrument. If you prefer to use the data from a Thermo Q-Exactive Plus, see the version of this tutorial entitled Skyline DIA QE.

In this tutorial we will learn how to use Skyline to perform targeted post-acquisition analysis for peptide and inferred protein detection and quantification using a SWATH-MS dataset acquired on a QqTOF instrument (6600 TripleTOF, Sciex) using an 64 variable width window precursor isolation scheme and a 1 hour gradient.

The data are from samples replicating the LFQBench study where quantitative benchmarking samples were created by mixing proteomes of 3 organisms in defined ratios (figure).

Initially, we will set all the parameters in the Skyline session required to work with data-independent datasets and then we will proceed to extract the quantification information from the raw data files. We will import DDA search results to create a spectral library in order to generate peptide query parameters to analyse the DIA data.

[[figure adapted from Navarro, P. et al. A multicenter study benchmarks software tools for label-free proteome quantification. Nat Biotech 34, 1130–1136 (2016)

]

# Getting Started

To start this tutorial, download the following ZIP file:

[zip file link]

Extract the files in it to a folder on your computer, like:

C:\Users\brendanx\Documents

This will create a new folder:

C:\Users\brendanx\Documents\MethodRefine

If you have been using Skyline prior to starting this tutorial, it’s a good idea to revert Skyline to its default settings. To do so:

- Start Skyline.

- From the  shaded blue tile on the Start Page, click Blank Document which looks like this:

- From the Settings menu, click Default.

- Click No on the form to save current settings

The settings in this instance of Skyline have now been reset to the default.

Since this tutorial covers a proteomics topic, ensure that the user interface control is set to the “Proteomics interface”

- Click the user interface control in the upper right hand corner of the Start Page and select Proteomics Interface which looks like this:

Skyline is operating in Proteomics mode which is displayed by the protein icon  in the upper right-hand corner of Skyline.

To tune for DIA analysis, we also want to be sure that all our transitions are integrated within Skyline. To do that, click Settings and choose Integrate all.

# Defining the data-independent acquisition settings and isolation scheme

We will set the parameters for extracting ion chromatograms from both MS1 and DIA-MS2 data in the following steps:

- From the Settings menu, click Transition Settings  and select the Instrument tab.

- Change the Max m/z value to “2000” m/z

- In the Transition Settings dialog box, click the Full-Scan tab

- Enter the settings as indicated in the screen shot below.

Note: The resolving power depends on the type and settings of the instrument used for data acquisition. The optimum will be slightly different for each dataset. In this analysis we are using centroided data to save space so we will select ‘centroided’ and specify a mass accuracy for extraction. With profile mode data the resolving power of the instrument can be specified.

Now we need to define a new isolation scheme according to the parameters defined on the instrument for data-independent acquisition.

Note: In this example, we used 64 variable width windows that covered the range from 400 to 1200 m/z with 1 m/z overlap.

- Still on the Full-Scan tab of the Transition Settings dialog box, click the Isolation scheme menu and select Add..

- In the Name field of the Edit Isolation Scheme form, enter “SWATH64”.

- Click the Prespecified isolation windows radio button.

- At the bottom of the form, check the Specify Margins box.

- Ensure the dropdown under Prespecified isolation windows that Measurement is selected.

- Open the file 64_variable_windows.csv  in Excel (e.g. from C:\Tutorials\4-DIA\TTOF\DIA).

The first column of the table is the start and the second column is the end m/z of each isolation window. The third colum specifies the margin = 0.5

Note: As the quadrupole transmission windows are not perfectly square, the margin option allows to specify how much of the window edges schould not be used for extraction. Skyline will then extract from start+margin until end-margin

Note:

- Highlight the relevant columns and copy the excel table with ctrl-c and paste it with ctrl-v into the Skyline table for isolation windows.

- The Edit Isolation Scheme window should now look like this:

- Click the Graph button to see how the isolation windows cover the specified range.

- Click Close.

In the Edit Isolation Scheme form, click OK.

- Make sure that the Isolation scheme that you have just created is selected in the Isolation scheme drop down menu in the Transitions Settings, Full-Scan tab.

- Also, in in the Full-Scan tab, make sure that the Retention time filtering is set to Use only scans within 5 minutes predicted RT. This is used for setting a 10 minutes window around the predicted RT.

- Go to Filter tab of the Transitions Settings dialog box.

Fill in the options according to the screenshot below:

- Go to the Library tab (if closed, the Transitions Settings window is under the Settings in the main Skyline menu)

- Ensure that the If a library spectrum is available, pick its most intense ions is enabled and enter “6” in Pick: product ions.”

- Select the From filtered product ions radio button.

The Library tab of the Transition Settings for should look like this:

- Click OK.

- Save your Skyline document as DIA_TTOF_acquisition_scheme.sky

# Adding query parameters for the iRT peptides

A set of 11 synthetic peptides with well characterized chromatographic behavior have been spiked into the samples for the purpose of making a linear regression of the measured retention times of these peptides and the ‘iRT values’ [see Escher, C. et al. Using iRT, a normalized retention time for more targeted measurement of peptides. Proteomics 12, 1111–21 (2012)]. This allows us to project the iRT values for the target peptides onto the retention time space of the DIA runs facilitating accurate predictions of their retention times in each DIA run we will analyze. We will first add the query parameters for the iRT peptides to the Skyline document.

- Open the Biognosys_iRT_for_OpenSWATH_6tr.tsv file in Excel or a text editor and notice the information contained.

- In Skyline, click File and then Import.

- From the fly-out menu, select Transition List.

- Select the file Biognosys_iRT_for_OpenSWATH_6tr.tsv and click Open.

- Skyline should ask if you want to create new iRT calculator. Click Skip. We will create the iRT calculator in a separate step later.

- Skyline should now ask if you want to create a spectral library from the spectral library intensities. Click Create.

- You should now see the 11 iRT peptides on the left of the document. Click on the first peptide (LGG…) and click on the + beside this peptide to expand.

You should see the precursor and fragment ion m/z and the ion type annotations as well as their intensity rank in the spectral library. You should also see the pseudo-spectrum created by Skyline from the fragment ion relative intensities in the Library Match window (if you don’t see the spectrum click View and then select Library Match). There should be 1 protein, 11 peptides, 11 precursors, and 66 transitions as highlighted in the bottom right corner of the screenshot below:

# Creating a spectral library

We will import some search results of DDA analysis of one replicate each of the A and B samples in order to generate a spectral library and peptide query parameters to query the DIA data. The DDA search has already been performed by us using the Comet search engine and post processed using PeptideProphet – see this link for more DDA search engines supported by Skyline https://skyline.ms/wiki/home/software/Skyline/page.view?name=building_spectral_libraries ). We will start from the interact.pep.xml file which is the output of PeptideProphet and contains the database search results from both DDA files.

- From the Settings menu, click Peptide Settings and click the Library tab.

- Click the Build button and add “DIA_TTOF_tutorial_library” in the Name.

- You may have to click Browse and set the Output Path to the current directory.

- Set the “Cut-off score set to “0.95” and select Biognosys-11 (iRT-C18) from the iRT standard peptides drop-down menu

Note: 0.95 is the threshold applied to the PeptideProphet probability computed for every peptide spectrum match in the DDA database search – in this particular data set this corresponds to a PSM false discovery rate of 0.2% but this will differ among data sets so a score threshold to achieve the FDR you want to use should be entered here.

Your Build Library form should looks something link this:

- Click Next

In the Build Library form that appears:

- Click Add Files and navigate to C:\DIA_LFQB\TTOF\DDA_search in your QE directory

- Clink on  interact.pep.xml in the Add Input Files dialog box (Note: you might see interact.pep, if the extension is not shown) and then click Open.

- Click Finish to start the library build process. The library will take 1-2 minutes to build.

Skyline will report that “The library built successfully. Spectra matching the following peptides had multiple ambiguous peptide matches and were excluded”

- Click “OK”

Skyline should now indicate that it was able to find the iRT peptides and could make a linear regression to generate the iRTs fort the peptides added to the library

- Click on Success (highlighted in blue) to see the linear regression

- Click Close and OK.

Skyline will ask if you want to recalibrate the iRT standard values.

- Click No.

Skyline should ask if you want to add a Retention Time Predictor.

- Change the time in Time window to “4” minutes and click Yes, then click OK to close the form.

- You can visualize the spectral library by selecting Spectral libraries from the View menu.

- Save the file as “DIA_TTOF_library.sky.”

# Adding Targets

We will add 12 target proteins (4 from each species) to extract from the DIA data by adding the protein sequences in FASTA format to Skyline.

- From the File menu, select Import and then FASTA.

- Select the target_protein_sequences.fasta file from the C:\DIA_LFQB\TTOF\DIA  directory.

Skyline should show you the following indicating that for one of the proteins in the fasta file no peptides were present in the spectral library.

- Click Remove.

You should now have a document containing 12 proteins, 148 peptides, 178 precursors, and 1,044 transitions as indicated in the lower right hand corner.

# Adding Decoys

The peptide query parameters only contains target peptides and proteins. Decoys need to be generated in the next step.

- From the Refine menu, select Add Decoys.

  - Leave the default number of decoy precursors which is 136. This is the same as the number of precursors in the document not counting the iRTs.

  - Select Shuffle Sequence from the Decoy Generation Method dropdown menu.

  - Click OK.

- Save the file as “DIA_TTOF_targets.sky”

# Performing the DIA data analysis

- From the File menu select Import and then Results.

- In the Import results form, choose Add single-injection replicates in files.

- At the bottom of the form, select Many from the “Files to import simultaneously menu

- Click the Show chromatograms during import checkbox.

- Click OK to close the form.

- In the Import Results Files, drag over and select all 6 DIA files from C:\DIA_LFQB\TTOF\DIA

  - collinsb_X1803_171.mzXML

  - collinsb_X1803_172.mzXML

  - collinsb_X1803_173.mzXML

  - collinsb_X1803_174.mzXML

  - collinsb_X1803_175.mzXML

  - collinsb_X1803_176.mzXML

- Click  Open.

Skyline should ask if you want to remove the file name prefix.

- Click “Do not remove.”

The next tasks in Sample Annotation, can be performed while the data is importing.

Note: The DIA data should start importing and the target and decoy transitions are extracted. This process can take some time (~5 minutes). For this tutorial, we selected data that was already converted from the raw data format to mzXML and centroiding was performed (using msconvert with vendor centroiding). Using centroided data increases processing speed compared to profile data. Furthermore we use a small peptide query parameter set to make the processing time manageable for the course. In a lot of applications larger peptide query parameter sets are chosen resulting in a longer time for the data extraction process.

# Sample Annotation

Here we need to define which samples belong to which experimental group

- From Settings, select Document Settings and then choose Annotations to bring up a form to add one annotation to the documents.

- Click Add to see the Define Annotation window.

- In the Name field, enter “Condition.”

- In the Type field, choose Value List.

- In the list under Applies To, choose Replicates.

In our experiment we have two conditions: Condition A are samples that have chimeric proteome composition of E.coli 20%, yeast 15% and human 65%, and condition B with composition of  E.coli 5%, yeast 30% and human 65%.

The define Annotation window should like this:

- Click “OK”

- Follow the steps above and create a second annotation named “BioReplicate.”

- Select Test in the Type menu for the BioReplicate annotation.

- Click OK.

Skyline will now return you to the Document Settings window and be sure you are still looking at the Annotations tab.

- Click the checkbox to select Condition.

- Click OK.

- From the View menu, choose Document Grid which brings up the Document Grid: Proteins window.

- In the Document Grid: Proteins window, click the Views manu and from the drop-down menu select Replicates.

- Annotate the samples as shown in the screen capture below:

- Close the Document Grid: Replicates window by clicking on the red X in the upper right hand corner of the form.

For easy viewing we will split the data by condition into 2 windows.

- From the View menu, choose  Arrange Graphs and from the submenu, Grouped.

- In the Arrange Graph Grouped dialog box, enter “2” for Group panes

- Click the radio button for Distribute graphs among groups

- Under the Display heading, choose Tiled.

- The Sort order option should read:  Document.

- Click OK.

Note: If the data is still importing at this stage you will need to wait until this is finished to save the document.

- Save your skyline document as DIA_TTOF_extracted.sky

# mProphet

In this section, we will apply the mProphet algorithm to the data. The best scoring peak groups are selected and the q-values are calculated to enable FDR control.

- From the Refine menu, choose Reintegrate

- In the Reintegrate form that appears, select: Add in the Peak scoring model dropdown.

- Click Train Model and inspect the model score distributions

- Enter “DIA_TTOF_tutorial” in the Name Field.

The window should look something like like this (the score distribution and weights might be slightly different depending on the Skyline version or minor difference in extraction parameters):

- Inspect the target and decoy discriminant score distributions in the “Model Scores” tab.

- Click on the Feature Scores tab and then select each of the scores in the table one by one. Notice which scores are best at discriminating between targets and decoys.

- Inspect the P Values and Q Values tabs. Are the majority of the targets detected?

- Click OK.

- Back in the Reintegrate window select following:

- Click OK.

- From the Settings menu choose Integrate all to quantify using all transitions

- Save your skyline document as DIA_TTOF_mProphet.sky

# Inspect the data manually

Now we will manually inspect some of the chromatography and underlying spectra.  To do that, we need to be sure that the Retention Times and Peak Area views are showing:

- From the View menu click Retention Times and then choose Replicate Comparison.

- Again from the View menu, click Peak Area and Replicate Comparison from the submenu.

Now both views should be visible.  Let’s configure them optimally on the screen:

- If these new windows are floating you can dock them by clicking on the top border of the floating window, holding the left mouse button down, and dragging this window:

- You can also similarly dock the Library Match window as shown below.

- Similarly, dock the Peak Area: Replicate Comparison and Library Match windows so that all information is easily viewable, as below:

- Click on the first E. coli protein: sp|P63284|CLPB_ECOLI.

You should see all of the peptides for this protein shown on the various plots (XIC chromatrography, peak areas, and retention time replicate graphs). The screenshot above is an example of when one protein is selected and all of the peptides for this protein are summarized in each of these views (except library match window where nothing is shown).

From the peak area replicate comparison, does this protein appear to be differentially regulated?

- If you select this fifth peptide (LPQVEGTGGDVQPSQDLVR) in this protein, then you get specific information for this peptide in all of these views.

- Examine the peak area patterns for the rest of the peptides belonging to this protein. Is the quantitative pattern for the peptides from this protein consistent with the expected differential regulation pattern?

NOTE: If there is more than one precursor charge state for a given peptide sequence these are extracted and scored separately. You can look at these by clicking the + next to the peptide sequence and clicking on the individual charge states.

- Click on some of the human proteins in the document. Examine the replicate peak areas from the protein level view and the peptide level view. Are the peak areas consistent with the expected ratios? What about the yeast proteins?

- Click again on the LPQVEGTGGDVQPSQDLVR peptide in the first protein

- Notice than if you hover the cursor over the chromatography a circle appears. If you click the circle, Skyline will open the full scan spectrum zoomed on the y8 ion that you selected. You can click the magnifying glass to zoom out and look at the full DIA scan.

- Explore the data further manually (including some decoys).

# Mass and retention time deviation

We can examine the mass accuracy and retention time prediction accuracy to determine if the optimal extraction parameters have been used.

- From the View menu, choose Mass errors and then Histogram. to the see the distribution of mass errors over the data set. Could the extraction window (±20 ppm) have been further optimized?

To the see the linear regression used to predict the target peptide retention times based on the iRT peptides and library iRT values from the target peptides:

- From the View menu, choose Retention Times and then Regression.  Lastly choose, Score to Run.

- Right click on the graph and select Plot and then Residuals to see the deviations from the predicted retention times in this data set. Could the extraction window (± 5 minutes) have been further optimized for this analysis?

Note: As the spectral library for this analysis was constructed from a side-by-side analysis of the same samples. As such, the accuracy of the retention time predictions are very good. Retention times from external spectral libraries acquired on different instruments, at different times, from different samples would lead to larger errors in these predictions.

- Save the file as DIA_TTOF_inspected.sky

# Quantitative comparison

- From the View menu, select Group Comparison and then Add.

- Enter the parameters of the group comparison as in the screenshot below (Note: you need to click Advanced to see lower options):

- Click OK

From the base screen, click the View menu and then select initially Group Comparison, followed by species_comparison from the submenu.

A table should appear that shows the peptide level fold-change and adjusted p-value for the comparison between the A and B groups.

- Expand the width of the Protein column header so that you can see the full protein names – including the corresponding species name.

- Right-click on the header of the Fold Change Result column and click on Sort Ascending.

Inspect the fold changes produced for some of the peptides in the table keeping in mind which species they are from and the expected ratios. What about the adjusted p-values?

- Click on Bar Graph in the top left corner of the table.

A bar graph of the fold changes for these peptides appears. You can highlight certain peptides in the graph by clicking on various peptides in the Targets window of the Skyline document (so that you can confirm which species various peptides in the graph are from).

- Right-click on the graph and select Settings to re-open the Edit Group Comparison window.

- Change the Scope option from Peptide to Protein.

- Click on Bar Graph to see the protein level fold change.

- Now you can see from which species the proteins are from in the x-axis labels. Do the measured ratios fit the expected ratios well? Are there some proteins missing?

ome

- On the Edit Group Comparison enable Use zero for missing peaks and look

- Close the Edit Group Comparison window.

- Save the document as “DIA_TTOF_group_comparison.sky”

- Export the data for later use in MSstats by clicking on the File menu and select Export and then choose Report.

- From the External Tools folder, select MSstats Input.

(Note: MSstats must already be installed as an external tool to have access to this report).

- Save as “DIA_TTOF_LFQB_MSstats_Input.csv”

# Comparing MS1 and MS2 data

Until now we have only been looking at extracted ion chromatograms from MS2 data (i.e. fragment ions). Now we will compare the signals from MS1 XICs and MS2 XICs.

- From the Settings menu, click on Transition Settings  and then choose Filter.

- In the Ion types field add “p” to the ion types (indicating precursor).

The MS1 XICs from the precursor ions will now be overlaid with the DIA MS2 level fragment ion XICs from the fragment ions.

In general, the precursor ions have a much higher absolute intensity than the fragment ions. As such, it is usually easier to view these in separate graphs:

- Click the View menu and select Transitions and then Split Graph.

Now the precursor XICs from the MS1 scans are displayed in the upper graph and the fragment ion XICs from the DIA scans are displayed in the lower graph.

- Search (using ctrl-F) for the peptide SDTAAAAVR and examine the difference between the MS1 and MS2 data.

- Browse through some peptides and compare the signals coming from the MS1 XICs and the DIA MS2 XICs. Is there a difference in the selectivity between the MS1 and DIA MS2 data?

- Can you find cases where the DIA MS2 data is higher quality (i.e. better selectivity) than the MS1 data similar to peptide above? Can you find any cases where it is the opposite?

- Save the document as DIA_TTOF_MS1_comparison.sky

Congratulations! You have completed the tutorial